# What is LiDAR?

Measure distance using light

The idea is simple

Lidar is a device that emits pulses of laser light and monitors the time it takes for them to return after they hit an object.

The LiDAR device does not emit a continuous beam…

No, it emits very small and very fast pulses of light (like flashes).

# How LiDAR work?

It emits light → strikes an object → returns → we calculate the time → we know the distance

Laser transmission :the device emits a very fast laser pulse ,the pulse travels at the speed of light , The pulse collides with the body. The pulse hits a wall, a person, or a chair. Light returning: Part of the light returns to the device. Time Calculation The device measures time from:

Send → Return

The process is repeated. The lidar repeats this process thousands of times per second, changing the laser direction each time. Making a 3D Picture Each measurement = a point Group of points = Point Cloud In conclusion: We obtain a 3D map of the location

Distance calculation

d=)c×t​(\2

c = Speed ​​of light

t = Total time (outbound + return)

d = is the distance in a single direction.

-Accuracy depends on the "number of pulses.”

It doesn't just measure distance! LiDAR can determine object shape. Its boundaries and dimensions (height/width)

-It varies depending on the surface type: a white or reflective surface → Strong signal; a black surface → Weak signal; Glass → Light can pass through it.

-This is considered a defect Sensor Fusion (The Essential Solution)

Don't rely on LiDAR alone  Use other sensors:

Ultrasonic → Detects any nearby object (even glass and black)

Infrared (IR) → Supports close detection

-There are two main types:

Mechanical LiDAR

Rots 360 degrees

More accurate but slower and more expensive

Solid-State LiDAR

Has no rotating parts

Faster and smaller

-Why a laser specifically?

Because:

It's extremely fast (speed of light)

It's extremely precise

It's not affected by movement like a camera

# Basic components of a LiDAR system

The basic components of LiDAR

1 Laser Source (Emitter)

- Produces short pulses of laser light

- Acts as the “light transmitter”

- Sends thousands to millions of pulses per second

2   Scanning System

- Directs the laser beam across the environment

- Can rotate or use mirrors

- Allows 2D/360° or 3D scanning

3     Receiver (Photodetector)

- Detects the reflected laser light

- Very sensitive to weak signals

- Converts light into electrical signals

4  Timing System (Clock Unit)

- Measures time of flight (TOF)

- Calculates how long the laser takes to return

5 Processing Unit

- Processes raw signals

- Converts time measurements into distance

- Generates point cloud data (3D map)

6  Position & Motion System (GPS / IMU)

- GPS: provides global location

- IMU: measures orientation, tilt, and movement

- Helps build accurate maps in motion systems (like cars/robots)

7  Data Storage / Output Unit

- Stores scanned data

- Sends results to a computer or controller

- Used for visualization and mapping

Simultaneous Localization and Mapping (SLAM)?

The device does two things simultaneously:

It determines its location (localization).

It creates a map of the location (Mapping)

# Comparison with other sensors

1)LiDAR vs Ultrasonic Sensor

| Uses laser to measure distance | Uses sound waves |
| --- | --- |
| Very high accuracy | Lower accuracy |
| Affected by glass and dark surfaces | Works better with glass |
| Higher cost | Cheap and simple |

2) LiDAR vs Camera

| Generates 3D point cloud | Provides 2D image |
| --- | --- |
| Works in darkness | Affected by lighting |
| Does not understand visual content | Needs AI for object detection |

3) LiDAR vs Radar

| Weak in fog/rain | Uses radio waves |
| --- | --- |
|  | Works in all weather |
|  | Lower resolution |
|  | Cheaper |

4) LiDAR vs Infrared Sensor (IR)

|  | Short range |
| --- | --- |
|  | Cheap |
|  | Affected by sunlight |

# Common applications of LiDAR

1) Autonomous Driving

Uses LiDAR to understand the road around the vehicle.

Innovation:

Advanced car

Obstacles

Assists in driverless driving

2) Geographic Reading and Surveying

Creating 3D maps (3D maps)

Measuring geological features and terrain

Used in:

Google Maps

Scanning

3) Agriculture and Forestry

Studying plant reproduction

High-resolution mapping

Used in smart forest management

4) Urban Planning

Designing cities and buildings

Measuring data accurately

Aviation and infrastructure analysis

5) Archaeology

Underground debris or soil

Precisely mapping sites