# AUTONOMOUS MOVEMENT USING ROS

Architecture, Control Theory, and Implementation

---

# 1. Introduction

The Robot Operating System (ROS) is an open-source middleware framework that provides hardware abstraction, device drivers, libraries, visualization tools, message passing, and package management for robotics applications. ROS employs a distributed architecture based on a publish-subscribe messaging pattern, where software components (nodes) communicate via named topics. This decoupling enables modular system design, where sensors, algorithms, and actuators can be developed and tested independently before integration.

ROS has become the de facto standard for autonomous mobile robotics in research and industry. Its extensive ecosystem includes navigation stacks, SLAM implementations, perception pipelines, and simulation tools (Gazebo). The framework's message-based inter-process communication allows seamless integration of heterogeneous components—LiDAR sensors, wheel encoders, motion planners, and motor controllers—into a cohesive system. This standardization reduces development time and enables code reuse across different robotic platforms.

Two major ROS distributions are currently relevant: ROS 1 Noetic Ninjemys (final ROS 1 release, supported until 2029) and ROS 2 Humble Hawksbill (current LTS release). This report focuses on ROS 1 Noetic for compatibility with widely-deployed navigation stacks, though the architectural principles apply to both distributions.

---

# 2. Technical Deep Dive: `/cmd_vel` and `geometry_msgs/Twist`

## The `/cmd_vel` Topic

The `/cmd_vel` topic is the standard ROS interface for commanding robot motion. It receives velocity commands from navigation planners (move_base, local planners) and passes them to motor controllers. The topic uses the `geometry_msgs/Twist` message type, which encodes commanded linear and angular velocities in three-dimensional space.

## Message Structure

```

geometry_msgs/Twist:

Vector3 linear:

float64 x   # Forward velocity (m/s)

float64 y   # Lateral velocity (m/s)

float64 z   # Vertical velocity (m/s)

Vector3 angular:

float64 x   # Roll rate (rad/s)

float64 y   # Pitch rate (rad/s)

float64 z   # Yaw rate (rad/s)

```

For differential drive ground robots, only `linear.x` (forward/backward) and `angular.z` (yaw rotation) are relevant. All other fields are zero.

## Kinematic Derivation for Differential Drive

For a differential drive robot with wheelbase $b$ (distance between wheels), the relationship between commanded velocity and individual wheel speeds is:

$$v_L = v - \frac{\omega \cdot b}{2}$$

$$v_R = v + \frac{\omega \cdot b}{2}$$

Where:

- $v$ = commanded linear velocity (linear.x) in m/s

- $\omega$ = commanded angular velocity (angular.z) in rad/s

- $b$ = wheelbase in meters

- $v_L$ = left wheel linear velocity

- $v_R$ = right wheel linear velocity

**Derivation:** The robot rotates about an instant center of curvature (ICC). The right wheel travels a larger arc ($v + \omega b/2$) while the left wheel travels a smaller arc ($v - \omega b/2$). The speed difference $\omega b$ equals the difference in wheel velocities.

## Physical Intuition

| Command | Motion |

|---------|--------|

| $v = 0$, $\omega \neq 0$ | Pure rotation in place (zero pivot) |

| $v \neq 0$, $\omega = 0$ | Straight-line motion |

| $v \neq 0$, $\omega \neq 0$ | Arc motion (curved path) |

When $\omega > 0$, the robot turns left (counterclockwise). When $\omega < 0$, the robot turns right (clockwise).

---

# 3. The Navigation Pipeline

The ROS navigation pipeline performs hierarchical motion planning from perception to actuation:

1. **Perception Layer**: Sensor inputs (LiDAR, IMU, wheel encoders) are processed and fused. The costmap_2d package constructs a 2D occupancy grid representing free space, obstacles, and inflation zones.

2. **Planning Layer**: The global planner (nav_fn or global_planner) computes a collision-free path from start to goal using A* or Dijkstra's algorithm on the costmap. The local planner (DWA or TEB) generates velocity commands that follow the global path while avoiding dynamic obstacles not in the map.

3. **Actuation Layer**: The local planner outputs `/cmd_vel` messages. The diff_drive_controller or hardware driver converts these to wheel velocities using the kinematics above. Odometry feedback closes the loop.

The move_base node orchestrates this pipeline, managing global planner, local planner, costmaps, and recovery behaviors. The complete pipeline: `sensor fusion → costmap → global planner → local planner (DWA) → /cmd_vel → motor controllers → robot motion → odometry feedback`.

---

# 4. System Flow Diagram

flowchart LR
    A[LiDAR Scan] --> B[Costmap Update]
    B --> C[Global Planner]
    C --> D[Local Planner DWA]
    D --> E[/cmd_vel]
    E --> F[Motor Controllers]
    F --> G[Robot Motion]
    G --> A

---

# 5. Implementation (Pseudo-code & Node Structure)

To simulate the autonomous movement logic theoretically, the following structure outlines a ROS node designed for obstacle avoidance.

## A. Logic Pseudo-code (The 'Stop and Spin' Strategy)

```

FUNCTION listener_callback(scan_data):

// 1. Perception: Data Processing

GET ranges from scan_data for indices -30° to +30° [cite: 73, 76]

FILTER out 'inf' or 'nan' values [cite: 79]

FIND minimum_distance in the filtered ranges

// 2. Decision: Planning Logic

IF minimum_distance < 0.5 meters: [cite: 76, 77]

SET linear_velocity = 0.0  (STOP) [cite: 77]

SET angular_velocity = 0.5 (SPIN) [cite: 77]

LOG "Obstacle Detected - Spinning" [cite: 78]

ELSE:

SET linear_velocity = 0.2  (MOVE FORWARD) [cite: 77]

SET angular_velocity = 0.0 (STRAIGHT) [cite: 77]

LOG "Path Clear - Moving Forward" [cite: 78]

// 3. Actuation: Execution

PUBLISH velocity_commands to /cmd_vel [cite: 74]

END FUNCTION

```

## B. ROS Node Architecture

The system follows a standard modular structure:

| Component | Specification |

|-----------|---------------|

| Node Name | `obstacle_avoidance_node` |

| Subscribed Topic | `/scan` (Type: `sensor_msgs/LaserScan`) to perceive the environment |

| Published Topic | `/cmd_vel` (Type: `geometry_msgs/Twist`) to command movement |

| Execution Rate | 10 Hz (The decision loop runs every 0.1 seconds) |